DNA extraction protocol (adjusted after ZymoBIOMICS™ DNA Miniprep Kit)

1. Add 750 µl ZymoBIOMICS™ Lysis Solution to a ZR BashingBead™ Lysis Tubes (0.1 & 0.5 mm). Pick colony, add into tube, mix and cap tightly.
2. Secure tube in a vortex adapter and vortex 10 min at full speed.
3. Centrifuge the ZR BashingBead™ Lysis Tubes in a microcentrifuge at ≥ 10,000 x g for 1 minute.
4. Transfer up to 400 µl supernatant to the Zymo-Spin™ III-F Filter in a Collection Tube and centrifuge at 8,000 x g for 1 minute. Discard the Zymo-Spin™ III-F Filter.
5. Add 1,200 µl of ZymoBIOMICS™ DNA Binding Buffer to the filtrate in the Collection Tube from Step 4. Mix well.
6. Transfer 800 µl of the mixture from Step 5 to a Zymo-Spin™ IICR Column in a Collection Tube and centrifuge at 10,000 x g for 1 minute.
7. Discard the flow through from the Collection Tube and repeat Step 6.
8. Add 400 µl ZymoBIOMICS™ DNA Wash Buffer 1 to the Zymo-Spin™ IICR Column in a new Collection Tube and centrifuge at 10,000x g for 1 minute. Discard the flow-through.
9. Add 700 µl ZymoBIOMICS™ DNA Wash Buffer 2 to the Zymo-Spin™ IICR Column in a Collection Tube and centrifuge at 10,000 x g for 1 minute. Discard the flow-through.
10. Add 200 µl ZymoBIOMICS™ DNA Wash Buffer 2 to the Zymo-Spin™ IICR Column in a Collection Tube and centrifuge at 10,000 x g for 1 minute.
11. Transfer the Zymo-Spin™ IICR Column to a clean 1.5 ml microcentrifuge tube and add 100 µl (50 µl minimum) ZymoBIOMICS™ DNase/RNase Free Water directly to the column matrix and incubate for 1 minute. Centrifuge at 10,000 x g for 1 minute to elute the DNA.
12. Place a Zymo-Spin™ III-HRC Filter in a new Collection Tube and add 600 µl ZymoBIOMICS™ HRC Prep Solution. Centrifuge at 8,000 x g for 3 minutes.
13. Transfer the eluted DNA (Step 11) to a prepared Zymo-Spin™ III-HRC Filter in a clean 1.5 ml microcentrifuge tube and centrifuge at exactly 16,000 x g for 3 minutes.
The filtered DNA is now suitable for PCR and other downstream applications.

Full protocol: https://files.zymoresearch.com/protocols/_d4300t_d4300_d4304_zymobiomics_dna_miniprep_kit.pdf