Theoretical and experimental investigation of Dopant-Free Hole-Transporting Materials for Stable and Efficient Perovskite Solar Cells

Group for Molecular Engineering of Functional Materials

Prof. Mohammad Khaja Nazeeruddin

Laboratory for Computational Molecular Design

Prof. Clemence Corminboeuf

Description of the Project

Due to their high power conversion efficiency (PCE), solution processability, and relatively low production cost,1,2 organic-inorganic metal halide perovskite solar cells (PSCs) are tremendously attractive. Efficient light absorbing perovskites such as methylammonium lead iodide (MAPbI3), exhibit wide absorption ranges over the complete visible solar spectrum.3 While bare perovskite materials display ambipolar transport, high charge carrier mobility, and long charge carrier lifetimes that facilitate the design of hole/electron transport layer (HTL/ETL) free PSCs,4–6 their corresponding PCE have not exceeded 14%. On the other hand, PSCs containing hole transport layers have reached PCEs over 20%.7 Clearly, HTLs are highly beneficial, if not necessary for high-performing PSCs8,9 making it crucial to design and test suitable hole transport materials (HTMs).10,11

Figure 1. The structure of a perovskite solar cell device and the current state-of-the-art HTM Spiro-OMeTAD.

Since Grätzel and Park’s incorporation of Spiro-OMeTAD into the first solid-state PSC in 2012,12 it has become the prevalent HTM choice. However, its high synthetic cost (~$600g–1) and complicated purification procedure prevent large-scale commercial production (see Figure 1).13,14 In addition, pristine Spiro-OMeTAD suffers from relatively poor hole mobility (μ=2×10-5–2×10-4cm2V-1s-1).15–18 In practical PSC device fabrication, Spiro-OMeTAD and other HTMs with low hole mobilities are usually doped with ionic additives such as bis- (trifluoromethylsulfonyl)imide lithium salt (Li-TFSI) to increase their conductivity and enhance the overall PSC performance.16,19 Yet these additives are detrimental to the long-term stability of PSCs due to their deliquescent nature.20 Consequently, dopant-free HTMs with high hole mobility arise as an appealing alternative, affording high PCE without simultaneously sacrificing the device stability.14,20–26

In this project, you will spend first several weeks in Prof. Clemence Corminboeuf group (LCMD) to learn how to compute hole mobility, the key property for dopant-free HTMs, using multiscale simulations combining classical molecular dynamics simulations, kinetic Monte Carlo techniques and electronic structure computations. You need to understand the theory governing the hole hopping mechanism and try to search the underlying structure-property relationship. After LCMD, you will go to Prof. Mohammad Khaja Nazeeruddin group (GMF). For experimental part, you will learn how to synthesis HTM molecules and how to fabricate a perovskite solar cell device. Finally, you understand both experimental and theoretical details and are able to interpret the comparison between theoretical prediction and experimental measured results.

References

1	M. A. Green, A. Ho-Baillie and H. J. Snaith, Nat. Photonics, 2014, 8, 506–514.

2	S. T. Williams, A. Rajagopal, C. C. Chueh and A. K. Y. Jen, J. Phys. Chem. Lett., 2016, 7, 811–819.

3	M. Grätzel, Nat. Mater., 2014, 13, 838–842.

4	L. Huang, X. Sun, C. Li, R. Xu, J. Xu, Y. Du, Y. Wu, J. Ni, H. Cai, J. Li, Z. Hu and J. Zhang, Sol. Energy Mater. Sol. Cells, 2016, 157, 1038–1047.

5	H. Zhou, Y. Shi, Q. Dong, H. Zhang, Y. Xing, K. Wang, Y. Du and T. Ma, J. Phys. Chem. Lett., 2014, 5, 3241–3246.

6	S. D. Stranks, G. E. Eperon, G. Grancini, C. Menelaou, M. J. P. Alcocer, T. Leijtens, L. M. Herz, A. Petrozza and H. J. Snaith, Science (80-. )., 2013, 342, 341–344.

7	L. Dongxue and Y. Liu, J. Semicond., 2017, 38, 11005.

8	E. J. Juarez-Perez, M. Wuβler, F. Fabregat-Santiago, K. Lakus-Wollny, E. Mankel, T. Mayer, W. Jaegermann and I. Mora-Sero, J. Phys. Chem. Lett., 2014, 5, 680–685.

9	Z. H. Bakr, Q. Wali, A. Fakharuddin, L. Schmidt-Mende, T. M. Brown and R. Jose, Nano Energy, 2017, 34, 271–305.

10	W. Chi, P.-P. Sun and Z.-S. Li, Nanoscale, 2016, 8, 17752–17756.

11	M. Saliba, S. Orlandi, T. Matsui, S. Aghazada, M. Cavazzini, J.-P. Correa-Baena, P. Gao, R. Scopelliti, E. Mosconi, K.-H. Dahmen, F. De Angelis, A. Abate, A. Hagfeldt, G. Pozzi, M. Graetzel and M. K. Nazeeruddin, Nat. Energy, 2016, 1, 15017.

12	H.-S. Kim, C.-R. Lee, J.-H. Im, K.-B. Lee, T. Moehl, A. Marchioro, S.-J. Moon, R. Humphry-Baker, J.-H. Yum, J. E. Moser, M. Grätzel and N.-G. Park, Sci. Rep., 2012, 2, 591.

13	P. Ganesan, K. Fu, P. Gao, I. Raabe, K. Schenk, R. Scopelliti, J. Luo, L. H. Wong, M. Grätzel and M. K. Nazeeruddin, Energy Environ. Sci., 2015, 8, 1986–1991.

14	F. Zhang, X. Liu, C. Yi, D. Bi, J. Luo, S. Wang, X. Li, Y. Xiao, S. M. Zakeeruddin and M. Grätzel, ChemSusChem, 2016, 9, 2578–2585.

15	Y. Xue, Y. Wu and Y. Li, J. Power Sources, 2017, 344, 160–169.

16	T. Swetha and S. P. Singh, J. Mater. Chem. A, 2015, 0, 1–16.

17	D. Poplavskyy and J. Nelson, J. Appl. Phys., 2003, 93, 341–346.

18	H. Xi, S. Tang, X. Ma, J. Chang, D. Chen, Z. Lin, P. Zhong, H. Wang and C. Zhang, ACS Omega, 2017, 2, 326–336.

19	N. J. Jeon, J. Lee, J. H. Noh, K. Nazeeruddin, M. Grätzel, S. Il Seok, M. K. Nazeeruddin, M. Grätzel and S. Il Seok, J. Am. Chem. Soc., 2013, 135, 19087–19090.

20	J. Liu, Y. Wu, C. Qin, X. Yang, T. Yasuda, A. Islam, K. Zhang, W. Peng, W. Chen and L. Han, Energy Environ. Sci., 2014, 7, 2963–2967.

21	S. Kazim, F. J. Ramos, P. Gao, M. K. Nazeeruddin, M. Grä Tzel and S. Ahmad, Energy Environ. Sci. Energy Environ. Sci, 2015, 8, 1816–1823.

22	G.-W. Kim, G. Kang, J. Kim, G. Y. Lee, H. Il Kim, L. Pyeon, J. Lee and T. Park, Energy Environ. Sci., 2016, 9, 2326–2333.

23	Y. Liu, Q. Chen, H.-S. Duan, H. Zhou, Y. Yang, H. Chen, S. Luo, T.-B. Song, L. Dou, Z. Hong and Y. Yang, J. Mater. Chem. A, 2015, 3, 11940–11947.

24	F. Zhang, C. Yi, P. Wei, X. Bi, J. Luo, G. Jacopin, S. Wang, X. Li, Y. Xiao, S. M. Zakeeruddin and M. Grätzel, Adv. Energy Mater., 2016, 6, 1600401.

25	C. Huang, W. Fu, C. Z. Li, Z. Zhang, W. Qiu, M. Shi, P. Heremans, A. K. Y. Jen and H. Chen, J. Am. Chem. Soc., 2016, 138, 2528–2531.

26	J. Liu, Y. Wu, C. Qin, X. Yang, T. Yasuda, A. Islam, K. Zhang, W. Peng, W. Chen and L. Han, Energy Environ. Sci., 2014, 7, 2963–2967.

27	P. F. Barbara, T. J. Meyer and M. a. Ratner, J. Phys. Chem., 1996, 100, 13148–13168.

28	R. a. Marcus, Angew. Chemie Int. Ed. English, 1993, 32, 1111–1121.

29	H. Bässler, Phys. status solidi, 1993, 175, 15–56.

30	R. Coehoorn, W. F. Pasveer, P. A. Bobbert and M. A. J. Michels, Phys. Rev. B - Condens. Matter Mater. Phys., 2005, 72, 1–20.