# BAT2 EXERCISE: Composite beam

# (2 problems, 1 to solve, 2 to study)

### Problem 1, joist

### Data

In other words, a building floor consisting of a mixed slab, supported by joists and bedplates at the same level. The joists were pre-dimensioned as IPE 330 in S235 steel. The static system of the joists, 2 m apart, is a composite beam on two supports with a span of 10 m, see Figure 1.1.

The profiled sheets are laid and nailed to the joists, with the sheet interrupted. The sheets are Cofrastra 40/0.75, steel SE 320 G (fyp = 320 N/mm2 ), sheet height hp = 40 mm, b0 = 102.5 mm, Ap = 1184 mm2 . They form a diaphragm that stabilises the beams horizontally. Total thickness: hc = 100 mm.

Concrete: C25/30 (fck = 25 N/mm2 , Ecm = 34.4 kN/m2 ).

Reinforcement: welded mesh, diameter 4 mm, s = 100 mm, B500B steel (fsk = 500 N/mm2 ).

The design values for internal forces (under uniform loads, moment given at mid-span, shear force at supports) are as follows:

Construction stage:	MEd = 131.4 kNm

VEd = 58.8 kN

Final stage :	MEd = 269.2 kN

VEd = 104.2 kN

b0

b0

HEA 340 base

HEA 340 base

330

330

IPE 330 joist

IPE 330 joist

10.0 m

10.0 m

Figure 1.1 - Beam system, joist view

Figure 1.2 - Cross-section of a joist

### Questions

- Check the structural safety of joists at the construction stage, EE calculation

- Check the structural safety of the joists at the final stage, EP calculation

### Problem 2, continuous windchest over 3 bays (TGC11 § 10.7.3)

### Data

The building used in one of the BAT1 problems (TGC11 § 6.9.3) is shown in Figure 2.1 below.

We're going to look at the bed frames of an office floor. It is made up of a mixed slab, resting on joists and bedplates. The top soles of the joists and the base plates are at the same level (Fig. 2.2). It is assumed that the reaction of the joists on the bedplates corresponds to a uniformly distributed load. The characteristic values of the actions, as well as the load cases to be considered, are defined below. For your convenience, the values of the corresponding internal forces are also given. The participating width is determined according to the definition in EN 1994-1-1.

Home

Offices

Restaurant

Sale

Basements

Home

Offices

Restaurant

Sale

Basements

Figure 2.1 - Load-bearing structure of the building

Figure 2.2 - Mixed floor supports

### Questions

- Check the buckling and tipping criteria for these bed bases

- Determining internal stresses and checking structural safety at the construction stage

- Determine the internal forces at the final stage, for an EP calculation with redistribution

- Check structural safety at the final stage, EP calculation

- Check the deflections at the construction and final stages.