# Fundamentals of Electrical Circuits and Systems

# ‘Circuits’ Part

# Thursday January 23, 2020

# Family Name : …………………                 		First Name :………………………...

# Sciper : …………………

Important :

- The duration of the exam is 1.5 hours.

- The 2 problems are independent and can be solved in any order.

- Any answer given without units or with false units will be considered as erroneous.

- Use only A4 sheets.

- Number each sheet.

- Put your name on the top right of each sheet.

- Write very legibly, do not use a pencil or red ink.

Problem 1

Find the Norton equivalent of the following circuit

2 A

12 V

4 

8 

5 

a

b

Problem 2

A balanced three-phase load connected in delta is supplied by a three-phase source with a line voltage of 240 V, f = 50 Hz. Each load absorbs an active power of 6 kW with a power factor of 0.8 (inductive).

- Find the load impedance per phase

- Calculate the module of the line current

- Calculate the value of the capacitance to be put in parallel with each load impedance to compensate for the reactive power.