Course

Prepare PCB files for manufacturing

- Antenna design constraints

  - Only FR4 dielectric

  - Substrate thickness either 0.8mm or 1.6mm

  - Maximum antenna size: 200x200mm

- Prep Model in HFSS

  - Alignment Targets for 2 layer designs

  - Screw holes for 3 or more layer designs

  - Edgecut outline

- Data generation in HFSS (See video)

  - Export dxf-file for every layer

  - Import this dxf file again in HFSS and export it as Gerber file

  - Open all Gerber files in KiCAD GerberViewer and check layers (alignment,size,etc)

  - Plot Gerber files to PDF

- Submit Gerber files and the printed PDF.