ENV – 418 RIVER ECOMORPHODYNAMICS AND BIOENGINEERING

Prof. Paolo Perona

Exercise session: vegetation dynamics and hydraulics

1. (COURSEWORK)

Figure 1 gives experimental data concerning the area of growth of a bacterial colony. The data points have been fit to the logistic equation,

yt=ab+e-c t,

where a, b, c, are positive constants.

1a. In terms of a,b,c what is the value of y(t) when t=0 and t=∞?

Figure 1

1b. Find the values of k1 and k2 for which the expression of y(t), given above, is a solution of the one-dimensional, non-linear, ordinary differential equation

dydt=k1y-k2y2,

Hint: Compute the derivative of y(t) and insert it in the ODE.

1c Find the fixed point (i.e. dy/dt=0) of the differential equation at 1b, and discuss them in relation to what you found at point 1a.

1d. In Figure 1 a=0.25 cm2, b= 0.005, and c=2.1 day-1. Find the numerical value of k1 and k2

1e. According to the diff equation in 1b and the values in 1d, what would be the area of growth as t∞ starting from an initial area of 100 cm2?

1f. Compute at which time the growth starts decelerating (i.e. the second derivative of y(t) is equal to zero) and the corresponding growth rate.

2.

Consider the logistic map

xn+1=αxn(1-xn),

as a discrete nonlinear model for population dynamics, specifically succession stages of vegetation on a river bar at discrete times. This model reproduces annual growth and periodic disturbances producing mortality. Simulate the behavior of the population for the following ranges of the parameter α:

1≤ α<3

3< α<1+61/2

1+61/2< α<a∞	, where a∞=3.5699

α > a∞

Describe in words the behavior that you observe and use the cobweb map technique to interpret it graphically.

3. (COURSEWORK)

Given the hydrologic data river bed flow time sequence (River_discharge.xlsx) convert it into water stages by means of the stage-discharge relationship Y=f(Q) (Stage-discharge_curve.xlsx). Use the plant roots GUI model in order to assess the root profile at the assigned river bank locations: 20 m, 40 m, 60 m and 80 m. The river section is also provided (River_section.xlsx) and the lowest elevation of the stream bed is 42.183 m.a.s.l.

Consider the case of herbaceous plant, and the following soil granulometry: d90=2.5 mm and d10=0.1 mm.

Recompute the root profiles and discuss the results under the following new conditions:

- Assume that the river hydrology has a vertical shift of –0.29 m3/s.

- Assume an associated change in sediment grain size distribution that changes the granulometry to d90=2.5 mm and d10=0.3 mm.

- Assume that the vegetation type has now evolved into "tree" together with the previous conditions at points 1 and 2.

4.

Vegetated vs non vegetated channel hydraulics.

Given the slope, i=1%, width b=10m, flowrate Q=15 m3/s and Manning coefficient n=0.025 sm-1/3  compute by means of the Chezy formula the critical and uniform flow depths, Y, shear stress 0 and mean velocity, U, Reynolds Re, and Froude, Fr, numbers and determine whether the stream flows under super or subcritical conditions. Assuming that juveniles vegetation grows in the channel as an array of plants with diameter d=1 cm and density equal to m=25 plants/m2, compute (by numerical iterations of Baptist’s formula) the new uniform flow conditions (water depth, velocity, shear stress Reynolds and Froude) and the riverbed roughness coefficient. Assume an actual drag coefficient of vegetation cD=1.5 and verify this assumption by using the diagram of Figure 1 below. Supposing that vegetation details are not known, use the current flow conditions in the vegetated bed to estimate the equivalent Manning coefficient of the vegetated channel reach.

Baptist’s formula for the Chezy coefficient Ck under vegetated bed conditions.

Ck=1nY162+CD∙m∙d∙Y2g		Cb=Y16n

Notice: the assumption concerning the drag coefficient should be a-posteriori verified using the drag chart for the rigid cylinder (see ahead).