# Exercise 5 – Questions

# Exercise 1 — Municipal solid waste separation

Municipal solid waste is separated by a mechanical sorting plant into a combustible fraction (30%) and another fraction (70%) that is digested by a subsequent biochemical process. Approximately half of the latter fraction is transformed to biogas during the anaerobic treatment.

- Draw the system as a quantitative flow chart.

- The concentrations of Hg are: Municipal solid waste input – 1.5 mg/kg, combustible fraction – 1 mg/kg, biogas – 0.005 mg/kg, digestion product – 3.4 mg/kg. Calculate all mass flows and transfer coefficients for Hg.

# Exercise 2 — Regional MFA of construction material

Figure 1 depicts a quantitative MFA of construction materials in a fictional region.

- Which stock will be most important for sand and gravel after 100 years, assuming that the material management stays constant?

- Which conditions are required in order that recycling of construction materials can make a substantial contribution to the supply of construction materials (both buildings and underground)?

- Which differences in material quality do you expect in the four stocks? Which is the fourth stock?

Figure 1: Diagram of the flows and stocks of construction materials in a fictional region (Brunner & Rechberger, 2005, pg. 257)

# Exercise 3 — Copper recycling

Assume that the incineration of one ton of municipal solid waste (copper content ca. 0.1%) yields the following solid residues: 250 kg of bottom ash, 25 kg of fly ash, 3 kg of iron scrap, and 3 kg of neutralisation sludge from the treatment of scrubber water. About 90% of the copper (Cu) leaves incineration via bottom ash, and 10% via fly ash. The Cu flow via other residues such as off–gas, iron scrap, etc. is <1% and can be neglected. Investigations show that by mechanically processing the bottom ash, approximately 60% of the Cu can be separated in the small fraction of metals concentrate. The Cu content of this fraction (ca. 50%) can be recovered in a metal mill.

- Develop a diagram of the waste management process described above.

- What is the recovery efficiency for the combined process "municipal solid waste and mechanical processing of bottom ash"?

# Exercise 4 — Photovoltaic cell production (solved in class)

The production of photovoltaic cells can be represented in a simplified manner depicting only the two main processes; silicon wafer production and photocell production. The flow data for each of the processes are as follows:

| Wafer production (for 1kg silicon wafers) |  |
| --- | --- |
| Input |  |
| Silicon [kg] | 1 |
| Energy [MJ] | 420 |
| Output |  |
| Silicon wafers [kg] | 1 |

| Photocell production (for 1m2 photovoltaic cells) |  |
| --- | --- |
| Input |  |
| Energy [MJ] | 13.6 |
| Glass [kg] | 10 |
| Silicon wafers [kg] | 1.6 |
| Output |  |
| Photovoltaic cells [m2] | 1 |

- Perform a system analysis: establish the system diagram and derive the balance equations.

- Develop the output equations considering each of the inputs, and calculate the amount of energy, silicon and glass needed to produce 1 m2 of photovoltaic cells.