SEMESTER EXERCISE – Minimally invasive tTIS project

Development of a minimally invasive tTIS system and its potential application.

Transcranial temporal interference stimulation (tTIS) offers a new way to non-invasively modulate deep brain regions by delivering multiple high-frequency electric fields that produce a low-frequency envelope at the region of interest, surpassing the depth-focality trade-off limitations of conventional transcranial stimulation techniques (e.g., Wessel, Beanato et al., 2023; for review Hummel & Wessel 2024). Yet, its clinical application is limited by low stimulation field strengths and spatial control, due to current shunting and dissipation at the scalp and skull, and safety restrictions on total current (Rampersad et al., 2019; Cassara et al., 2025)

Minimally invasive, subcutaneous implantable electrodes can address these barriers by allowing higher current amplitudes and more precise steering through closer proximity to the brain (Lee et al., 2022; Khatoun et al., 2021). Current advances in high-density flexible electrodes, miniaturized implantable pulse generators, and wireless data/power transfer systems provide the technological foundation for next-generation neurostimulation devices (Sarica et al., 2021; Stirling et al., 2021).

Assignment

Develop a technical concept for a minimally invasive, subscalp tTIS device designed to deliver steerable, multi-site deep brain stimulation. Your system should feature a strategic electrode array (~24–32 channels: stimulating and sensing), an implantable or semi-implantable pulse generator (iPG), and a safe, simple surgical approach. Identify major engineering challenges, such as electrode materials and miniaturization, as well as a clinical or research application that exploits the advantages of epidural tTIS over transcranial or intracranial solutions.

Challenges and Design Questions:

Electrode Configuration and Surgical Approach

Propose a solution for deploying a flexible, multi-electrode array subcutaneously, with minimal incisions. Configuration should enable programmable steering to multiple targets (e.g., striatum, hippocampus, thalamus, STN), supporting both stimulation and closed-loop sensing.

Electrode Materials and Longevity

Specify materials that are biocompatible and flexible, able to maintain properties for long term subcutaneous use. Distinguish optimal dimensions for both stimulating and recording electrodes.

Powering and Data Communication

Suggest engineering solutions for miniaturizing the iPG (similar to contemporary subcutaneous DBS iPGs), or leveraging wireless power and telemetry to external devices.

Address the challenge of continuous bidirectional data transfer for continuous sensing and closed-loop stimulation.

Clinical/Research Use-Case

Imagine a potential use case, leveraging the advantages of the designed system (wearable, continuous closed-loop deep brain stimulation).

References

Cassarà, Antonino M., Taylor H. Newton, Katie Zhuang, et al. 2025. “Recommendations for the Safe Application of Temporal Interference Stimulation in the Human Brain Part I: Principles of Electrical Neuromodulation and Adverse Effects.” Bioelectromagnetics 46 (2): e22542. https://doi.org/10.1002/bem.22542.

Hummel FC, Wessel MJ (2024) Non-invasive deep brain stimulation: interventional targeting of deep brain areas in neurological disorders. Nat Rev Neurol. Aug;20(8):451-452. doi: 10.1038/s41582-024-00990-8.

Khatoun, Ahmad, Boateng Asamoah, and Myles Mc Laughlin. 2021. “A Computational Modeling Study to Investigate the Use of Epicranial Electrodes to Deliver Interferential Stimulation to Subcortical Regions.” Frontiers in Neuroscience 15 (December). https://doi.org/10.3389/fnins.2021.779271.

Lee, Sangjun, Jimin Park, Da Som Choi, et al. 2022. “Feasibility of Epidural Temporal Interference Stimulation for Minimally Invasive Electrical Deep Brain Stimulation: Simulation and Phantom Experimental Studies.” Journal of Neural Engineering 19 (5): 056003. https://doi.org/10.1088/1741-2552/ac8503.

Rampersad, Sumientra, Biel Roig-Solvas, Mathew Yarossi, et al. 2019. “Prospects for Transcranial Temporal Interference Stimulation in Humans: A Computational Study.” NeuroImage 202 (November): 116124. https://doi.org/10.1016/j.neuroimage.2019.116124.

Sarica, Can, Christian Iorio-Morin, David H. Aguirre-Padilla, et al. 2021. “Implantable Pulse Generators for Deep Brain Stimulation: Challenges, Complications, and Strategies for Practicality and Longevity.” Frontiers in Human Neuroscience 15 (August). https://doi.org/10.3389/fnhum.2021.708481.

Stirling, Rachel E., Matias I. Maturana, Philippa J. Karoly, et al. 2021. “Seizure Forecasting Using a Novel Sub-Scalp Ultra-Long Term EEG Monitoring System.” Frontiers in Neurology 12 (August). https://doi.org/10.3389/fneur.2021.713794.

Wessel, Maximilian J., Elena Beanato, Traian Popa, et al. 2023. “Noninvasive Theta-Burst Stimulation of the Human Striatum Enhances Striatal Activity and Motor Skill Learning.” Nature Neuroscience 26 (11): 2005–16. https://doi.org/10.1038/s41593-023-01457-7.