Problems– Semiconductor devices II – Spring 2021 (Ionescu)

Problem 1: Subthreshold Leakage Current

An N-channel transistor has Vt=0.34V and S=85mV, W=10m and L=50nm.

- Estimate Ioff.

- Estimate Ids at Vg=0.17V.

Problem 2: Trade-off between Ioff and Ion

Does any of the following changes contribute to both leakage, Ioff, reduction and on current, Ion, enhancement?

- A larger Vt.

- A larger L.

- A shallower junction.

- A smaller Vdd.

Motivate your answer with equations, when possible.

Which of these reduce the Ioff without deteriorating the Ion?

Problem 3: Subthreshold Leakage Current and Threshold Voltage

Assume the gate oxide between an n+polysilicon gate and the p-substrate is 11 Angstrom thick and that Na=1E18.

a) What is Vt for this device?

b) What is the sub threshold swing, S?

c) What is the maximum leakage current if W=1um, L=18nm? (Assume Ids = 100W/LnA at Vg=Vt).

Vt Roll-off

Problem 4

Qualitatively sketch log(Id) vs. Vg (assume Vd=Vdd) for the following:

- L=0.2um, Na=1E15 (cm-3)

- L=0.2um, Na=1E17 (cm-3)

- L=1um, Na=1E15 (cm-3)

- L=1um, Na=1E17 (cm-3)

Please pay attention to the positions of the curves relative to each other and label all curves.

Problem 5

There is a lot of concern that we will soon be unable to extend Moore’s Law. In your own words explain this concern and the concern for high Ion and low Ioff.

- Answer this question using 1 paragraph of less then 50 words.

- Support your description in (a) with 3 hand drawn sketches of your choice.

- Why is it not possible to achieve high Ion and small Ioff by picking optimal Tox, Xj Wdep etc? Please explain in your own words.

- Provide three equations that help to quantify the issues discussed in part (c).