Problems. #6– Semiconductor devices II – Spring 2019 (Ionescu)

Problem 1: MEMS switches and resonators

Micro and Nano-Electro-Mechanical switches and resonators are devices that exploit the positional modulation of their movable parts by an electrical signal. Among the following properties, choose the correct ones:

- The pull-out Voltage of a two- or three-terminal mechanical relay can never be higher than pull-in Voltage (in absolute values) given that no charged layers are stacked in the gate stack.

- Both pull-in and pull-out events of a relay show, in terms of the transition slopes between OFF-to-ON and ON-to-OFF states, values that much below the 60mV/decade limit of a MOSFET that are only limited by the measurement setup.

- One key advantage of a MEMS or NEMS relay with in-series air-gap between the contacts is the independence of the leakage current in the OFF state on the operation temperature.

- The actuation voltage of a capacitive switch depends, in principle, on the dielectric material nature and thickness, in series with the air-gap and its thicknesses.

- A MEMS (passive) resonator is equivalent to a full series L, C, R circuit if one neglects any other substrate or contact parasitic.

- A DC voltage can be applied on a MEMS resonator across the air-gap in order to tune its equivalent stiffness.

- A DC voltage can be applied on a MEMS resonator across the air-gap in order to increase the capacitive coupling.

- Active MEMS resonators or Resonant Body Transistors) feature an intrinsic signal gain (in dB) that cannot be obtain in a vibrating conductive body structure.

- The quality factor, Q of a beam MEMS resonator is not affected by its environment (air pressure or vacuum level).

- The smaller the length of a suspended cantilever, the larger its resonant frequency.

Problem 2:

At the end of problems/questions #1 and #2:

- Enumerate five (5) major advantages of micro-electro-mechanical (MEM) switches compared to MOSFETs.

- Enumerate five (5) advantages of micro-electro-mechanical (MEM) switches compared to MIT switches.

- Enumerate five (5) big remaining technological and implementation challenges for electro-mechanical switches.

Problem 3: MEMS accelerometers

Micro and Nano-Electro-Mechanical accelerometers form a successful technology that entered into a broad range of applications, from wearables to automotive. Among the following properties, choose the correct ones:

- A MEMS accelerometer is normally based on a capacitive variation (due to gap or area change due to the displacement of a suspended mass conversion) into a voltage.

- A MEMS accelerometer has the disadvantage of high DC power consumption compared to solid-state sensors.

- In a MEMS structure with micrometer gaps used to detect acceleration with more than 10 but resolution, on a 1 – 10g scale, nanometer displacements (in the range of 10nm) are relevant (detectable)

- Inertial sensors for human activity and automotive share similar technology and some similar specifications.

- Similar MEMS technology to inertial accelerometers can be applied to build MEMS gyroscopes based on Coriolis force detection.

- SOI technology can be used to realize suspended MEMS structures by etching the buried oxide as a sacrificial layer.

- The role of protective package needed in MEMS accelerometer is to avoid any external electromagnetic interference in their operation.