Problems. #6– Semiconductor devices II – Spring 2019 (Ionescu)

Problem 1: MEMS switches and resonators

Micro and Nano-Electro-Mechanical switches and resonators are devices that exploit the positional modulation of their movable parts by an electrical signal. Among the following properties, choose the correct ones:

- The pull-out Voltage of a two- or three-terminal mechanical relay can never be higher than pull-in Voltage (in absolute values) given that no charged layers are stacked in the gate stack. TRUE. Slide 9.

- Both pull-in and pull-out events of a relay show, in terms of the transition slopes between OFF-to-ON and ON-to-OFF states, values that much below the 60mV/decade limit of a MOSFET that are only limited by the measurement setup. TRUE. No diffusion mechanism, but mechanical switching.

- One key advantage of a MEMS or NEMS relay with in-series air-gap between the contacts is the independence of the leakage current in the OFF state on the operation temperature. TRUE, we have an air band gap.

- The actuation voltage of a capacitive switch depends, in principle, on the dielectric material nature and thickness, in series with the air-gap and its thicknesses. TRUE. geff parameter depend on εr and the dielectric thickness gε. Slide 9.

- A MEMS (passive) resonator is equivalent to a full series L, C, R circuit if one neglects any other substrate or contact parasitic. TRUE. Slide 15.

- A DC voltage can be applied on a MEMS resonator across the air-gap in order to tune its equivalent stiffness. TRUE. Slide 14 and 15. Stiffness is 1/C, and depends on bias

- A DC voltage can be applied on a MEMS resonator across the air-gap in order to increase the capacitive coupling.TRUE, same as before.

- Active MEMS resonators or (Resonant Body Transistors) feature an intrinsic signal gain (in dB) that cannot be obtain in a vibrating conductive body structure.

- The quality factor, Q of a beam MEMS resonator is not affected by its environment (air pressure or vacuum level). FALSE. Vacuum reduce damping.

- The smaller the length of a suspended cantilever, the larger its resonant frequency. TRUE.

ωn= αn∙EImL4                 I= bh312  for a rectangular beam

Problem 2:

At the end of problems/questions #1 and #2:

- Enumerate five (5) major advantages of micro-electro-mechanical (MEM) switches compared to MOSFETs. Higher SS, Lower Ioff,, low power, no T dependence, modulate Q, higher Ion.

- Enumerate five (5) advantages of micro-electro-mechanical (MEM) switches compared to MIT switches. No T dependence, easier vertical SS to achieve,

- Enumerate five (5) big remaining technological and implementation challenges for electro-mechanical switches. Scaling down, hysteresis, CMOS integration, lower speed at non resonance frequency we need to apply DC voltage to actuate it (more than 1.5V)

Problem 3: MEMS accelerometers

Micro and Nano-Electro-Mechanical accelerometers form a successful technology that entered into a broad range of applications, from wearables to automotive. Among the following properties, choose the correct ones:

- A MEMS accelerometer is normally based on a capacitive variation (due to gap or area change due to the displacement of a suspended mass conversion) into a voltage. TRUE. Usually it is differential. Slide 37

- A MEMS accelerometer has the disadvantage of high DC power consumption compared to solid-state sensors. FALSE.

- In a MEMS structure with micrometer gaps used to detect acceleration with more than 10g but resolution, on a 1 – 10g scale, nanometer displacements (in the range of 10nm) are relevant (detectable)

- Inertial sensors for human activity and automotive share similar technology and some similar specifications. FALSE. E.g. power consumption, acceleration range.

- Similar MEMS technology to inertial accelerometers can be applied to build MEMS gyroscopes based on Coriolis force detection. I would say TRUE

- SOI technology can be used to realize suspended MEMS structures by etching the buried oxide as a sacrificial layer. TRUE

- The role of protective package needed in MEMS accelerometer is to avoid any external electromagnetic interference in their operation. I would say TRUE, but not only this.