Quiz 2

Name:

Date: 04.03.2025

Question 1

A) Give the expression of the intensity of light after passing through a layer of this material of thickness z as function of the initial intensity I0, thickness z, and absorption coefficient α:

II0,z,α= I0e-αz

B) What is the dimension of the absorption coefficient α ? [𝛼]=1/m

C) Let’s consider a material that can absorb light. It means that the refractive index consists of real and imaginary parts:

n=n+ik

Write the relation between the absorption coefficient α and the extinction coefficient k.

α=4πkλ

Question 2

Let’s consider an EM wave incident from half space with refractive index n1 to half space with refractive index n2 (see the picture below).

Which conditions are required in order that total internal reflectance occurs at the interface for sufficiently large θi? Choose the correct answer:

1) n1<n2

2) n1>n2

Question 3

Let’s consider a wave incident to the air – diamond interface (see the picture above ↑). Refractive index of air n1=1, refractive index of diamond n2=2.73.

A) What is the polarization of this wave (from the picture)

1) TE (also called S polarization)

2) TM (also called P polarization)

B) Estimate the value of the reflection coefficient r for incident angle θI≪1. The expressions for reflection coefficient for each polarization are given below.

Hint: Use the formula for the reflectance R=(nT-nI)2(nT+nI)2

R=2.73-122.73+12=1.7323.732=0.46 
For both polarizations