# Fusion and Industrial Plasma Technologies Course	Spring semester 2025

# List of questions for the oral exam

# Group 1 – Vacuum technologies and Industrial plasmas

- Define "base pressure" and "ultimate pressure in a vacuum system. Describe the process of "baking" and what it is that one seeks to accomplish by doing it.

- A vacuum chamber is connected via a rectangular duct of conductance "C" to a turbomolecular pump with pumping speed "S". Compute the effective pumping speed in terms of "S" and "C".?

- For an elastic collision between two bodies of mass m and M, find the maximum energy transfer fraction in terms of m and M. What is the implication for the temperatures of electrons and ions in a weakly-ionized plasma? Explain why this is an advantage for plasma chemistry reactors compared to thermal chemical reactors.

- Describe how you would derive Bohm's criterion for the ion velocity entering a plasma sheath.

- Describe how electrical breakdown in low-pressure gases is initiated, and how it develops, via breakdown, into a plasma.

- Explain how to calculate the sheath voltage drop to an electrically floating wall, for a DC plasma. Why are sheaths important for the microelectronics industry.

# Group 2 – Superconductivity and High current and high voltage power supplies

- What is a quench in a superconducting magnet and why do we need to protect a magnet against it? How is the protection done?

- A magnet can be wound by either layers or pancakes. What are the advantages/drawbacks of the two options?

- What is the reason to twist/transpose the superconducting strands (wires) in large cables, instead of assembling them straight/parallel?  Why is the forced flow cooling the preferred option for large fusion magnets?

- The toroidal field coils of ITER have a peak field of about 11.2T. Which problems would arise by increasing the magnetic field by 50%?

- What is the reversible and irreversible degradation for the ITER Nb3Sn conductors? What are the typical causes of the performance degradation in the superconducting magnets for fusion?

- What are the advantages of the HTS fusion coils compared to the LTS coils?  What, on the other hand, are the challenges and potential problems if a fusion coil is made of the HTS materials?

- What would be the scope of an R&D program for Demo power supplies?

- Explain the requirements for a fusion power plant to deliver 500MW to the grid.

# Group 3 - Millimeter wave systems and Cryogenics

- Explain the basic principles of a gyrotron; Why is it the preferred source for ECRH applications?

- Give a brief description of a Spectrum Analyzer and of a vector network analyzer; What are the main differences between the two types of devices? What factors determine the dynamic range?

- Describe the formalism that leads to the determination of eigenmodes in a rectangular waveguide. How would you apply that to circular waveguides?

- Derive the telegraphist equations for a transmission line with series resistance per unit length R, series inductance per unit length L, shunt conductance per unit length G, shunt capacitance per unit length C.

- Describe the different heat loss mechanisms in a cryogenic vessel and the means for minimizing the boil-off rate. Which additional losses are to be considered if the cryogenic vessel contains a superconducting magnet?

- A liquid-helium cryostat suddenly loses its insulating vacuum when an o-ring fails. Explain the chain of events that follows this “loss-of-vacuum” accident. Which heat-transfer mechanism instantly dominates? Why does the helium pressure rise so rapidly? What safety devices are present on the cryostat to deal with this type of event?

- Identify which material one would select in the design of a cryogenic apparatus for (i) a high-current coil stabilizer, (ii) low-heat-leak structural struts, (iii) an 80 K radiation shield, and (iv) small brackets or sensor mounts at 4 K. Give a brief justification for each choice.

# Group 4 – Materials, Control and data acquisition, Plasma Fuelling & Tritium Cycle

- Process(es) creating mechanical defect in a material under neutron bombardment.

- What are the criteria for selecting materials for DEMO ?

- Describe the main elements of the pellet injection system currently installed at JET.

- Discuss the general concerns with the tritium cycle, in particular the issue of the Tritium breeding ratios.

- To digitize a sinusoidal wave with 1% accuracy or better, one needs at least 32 samples per

period and a 6-bit precision. What are the digitization precision, frequency, and storage

required to acquire the combination of a dominant 8kHz wave and a -40dB secondary 512

kHz wave for 4s?

- The fastest acquisition system at TCV is a Guzik module acquiring 4 channels at 16 GHz

speed with 12-bit precision. Assuming TCP/IP transfer to analysis server via gigabit ethernet,

data processing speed of 64 MB/s, and intershot delay of 12 minutes, what is the maximum

acquisition duration that allows intershot analysis?

- Explain why the versioning of processed experimental data is not a simple matter. Do you

have a solution to propose?

- Can you explain the FAIR principles of open science and evaluate to which extent fusion data meets the corresponding requirements. What would you suggest to improve fusion data FAIRness?